ENCM 511 ASSIGNMENT #1

Author: Kyle Derby MacInnis

UCID: 10053959

Email: kderbyma@ucalgary.ca

Date: October 16, 2014

Professor: Dr. Smith

Date: 	_______________________________________________________________________

Signature:     _______________________________________________________________________

COMMENTS:

- The Code Snippets are generally snippets. For more information, please refer to the zipped assignment files.

- The Screenshots are one to a page.

- The Code Formatting was due to the CCES IDE.

Screenshots:

| 1 |  |  |
| --- | --- | --- |
|  | Figure 1 – The Coffee Pot has been added to the device, and the Base Address given. |  |
| 2 |  |  |
|  | Figure 2 – The First Coffee Pot has been Initialized and told to stay on – Waiting for Device to be Ready. |  |
| 3 |  |  |
|  | Figure 3 – The Device Ready Bit has been set, and the Coffee Pot is ready to begin making coffee. |  |
| 4 |  |  |
|  | Figure 4 – The Coffee Pot Power has been set on, and the Water is beginning to flow. |  |
| 5 |  |  |
|  | Figure 5 – Water is finished filling, water flow set to 0, and heat has been turned on, and the water is beginning to warm up. |  |
| 6 |  |  |
|  | Figure 6 - Water is beginning to bubble as temperature increases. |  |
| 7 |  |  |
|  | Figure 7 – The Water has finished heating up and the Coffee Pod has been initialized. |  |
| 8 |  |  |
|  | Figure 8 – The First Coffee Pot has finished (GUI Problem with Colour), and now the second pot is being prepared. |  |
|  | Figure 9 – Both Coffee Pots have finished and their pods have been initialized. (Issue with GUI Colour) |  |
| COFFEEPOT_MAIN.CPP |  |  |
| #include "MyCoffeePotFunctions.h" /* Managed drivers and/or services include */ #include "../system/adi_initialize.h" int main(void) { /* Initialize managed drivers and/or services */ adi_initComponents(); printf("Hello World from the Blackfin\n"); // Use mathematical paragraphs to separate ideas printf("\n"); printf("About to start my coffee pot simulator\n"); int numCoffeePots = 2; int whichDisplay = USE_TEXT_GUI ¦ USE_CCES_GUI; Init_CoffeePotSimulation(numCoffeePots, (WHICHDISPLAY) whichDisplay); COFFEEPOT_DEVICE *coffeePot1_BaseAddress = (COFFEEPOT_DEVICE *)NULL; COFFEEPOT_DEVICE *coffeePot2_BaseAddress = (COFFEEPOT_DEVICE *)NULL; coffeePot1_BaseAddress = Add_CoffeePotToSystem_PlugAndPlay(COFFEEPOT1, "WDS"); Do_Simulation_CPP(coffeePot1_BaseAddress); coffeePot2_BaseAddress = Add_CoffeePotToSystem_PlugAndPlay(COFFEEPOT2, "KDM"); Do_Simulation_CPP(coffeePot2_BaseAddress); } void USE_CCES_GUI_Delay(void) { int delay = 0xFFFF; for ( int count = 0; count < delay; count++) { // 0x1FFFFFF Seemed to work abeit slowly count = count + 1; } } |  |  |
| TURNONWATER_ASM.ASM |  |  |
| // LOCAL VARIABLE DECLARATIONS .section data1 // PROGRAM INSTRUCTIONS .section program //void TurnOnWater_ASM(COFFEEPOT_DEVICE *coffeePt_BaseAddress, unsigned char waterInputSpeed) { .global _TurnOnWater_ASM; // INPUT PARAMETERS #define PAR1_R0 R0 #define PAR2_R1 R1 // TEMPORARY REGISTERS #define TEMP_R2 R2 #define TEMP_R3 R3 // COFFEEPOT_DEVICE * BaseAddress #define COFFEEPOT_DEVICE_P0 P0 // CONTROL REGISTER OFFSETS #define CONTROLREGISTER_OFFSET 0 #define WATERINFLOWREGISTER_OFFSET 7 #define HEATERREGISTER_OFFSET 6 #define HEATERBOOST_OFFSET 8 #define LED_OFFSET 12 // CONTROL BITMASKS #define INITPOWERED (1 << 0) #define LEDPOWER (1 << 1) #define WATERPOWER (1 << 2) #define HEATERPOWER (1 << 3) #define DEVICEREADY (1 << 4) // LED BITMASKS #define LED1 (1 << (LED_OFFSET + 0)) #define LED2 (1 << (LED_OFFSET + 1)) #define LED3 (1 << (LED_OFFSET + 2)) #define LED4 (1 << (LED_OFFSET + 3)) #define LED_LED_ON LED2 #define LED_HEAT_ON LED3 #define LED_WATER_ON LED4 _TurnOnWater_ASM: LINK 20; // SELECT COFFEE POT BASE ADDRESS COFFEEPOT_DEVICE_P0 = PAR1_R0; // SET WATER FLOW RATE B[COFFEEPOT_DEVICE_P0 + WATERINFLOWREGISTER_OFFSET] = PAR2_R1; // TURN ON POWER AND WATER (AND LEDS) TEMP_R2 = W[COFFEEPOT_DEVICE_P0 + CONTROLREGISTER_OFFSET]; TEMP_R3 = WATERPOWER; TEMP_R2 = TEMP_R2 ¦ TEMP_R3; TEMP_R3 = LEDPOWER; TEMP_R2 = TEMP_R2 ¦ TEMP_R3; R3.L = lo(LED_WATER_ON); R3.H = hi(LED_WATER_ON); TEMP_R2 = TEMP_R2 ¦ TEMP_R3; R3.L = lo(LED_LED_ON); R3.H = hi(LED_LED_ON); TEMP_R2 = TEMP_R2 ¦ TEMP_R3; W[COFFEEPOT_DEVICE_P0 + CONTROLREGISTER_OFFSET] = TEMP_R2; // UPDATE DISPLAY .extern __Z23UpdateSimulationDisplayv; CALL __Z23UpdateSimulationDisplayv; CALL __Z23UpdateSimulationDisplayv; UNLINK; _TurnOnWater_ASM.END: RTS; |  |  |

TURNONHEAT_ASM.ASM

// LOCAL VARIABLE DECLARATIONS .section data1 // PROGRAM INSTRUCTIONS .section program // void TurnOnHeat_ASM(COFFEEPOT_DEVICE *coffeePot_BaseAddress, unsigned char heaterLevel, unsigned char heaterBoost); .global _TurnOnHeat_ASM; // INPUT PARAMETERS #define PAR1_R0 R0 #define PAR2_R1 R1 #define PAR3_R2 R2 // TEMPORARY REGISTERS #define TEMP_R3 R3 #define TEMP_R4 R4 // COFFEEPOT_DEVICE * BaseAddress #define COFFEEPOT_DEVICE_P0 P0 // CONTROL REGISTER OFFSETS #define CONTROLREGISTER_OFFSET 0 #define WATERINFLOWREGISTER_OFFSET 7 #define HEATERREGISTER_OFFSET 6 #define HEATERBOOST_OFFSET 8 #define LED_OFFSET 12 // CONTROL BITMASKS #define INITPOWERED (1 << 0) #define LEDPOWER (1 << 1) #define WATERPOWER (1 << 2) #define HEATERPOWER (1 << 3) #define DEVICEREADY (1 << 4) // LED BITMASKS #define LED1 (1 << (LED_OFFSET + 0)) #define LED2 (1 << (LED_OFFSET + 1)) #define LED3 (1 << (LED_OFFSET + 2)) #define LED4 (1 << (LED_OFFSET + 3)) #define LED_LED_ON LED2 #define LED_HEAT_ON LED3 #define LED_WATER_ON LED4 _TurnOnHeat_ASM: LINK 20; // SET COFFEE POT BASE ADDRESS COFFEEPOT_DEVICE_P0 = PAR1_R0; // TURN WATER OFF - **DOES NOT WORK WITH GUI //TEMP_R4 = ~WATERPOWER; //TEMP_R3 = TEMP_R3 & TEMP_R4; //R4.L = lo(~LED_WATER_ON); R4.H = hi(~LED_WATER_ON); //TEMP_R3 = TEMP_R3 & TEMP_R4; // TURN WATER FLOW RATE DOWN TO ZERO - **WORKS WITH GUI TEMP_R3 = 0; B[COFFEEPOT_DEVICE_P0 + WATERINFLOWREGISTER_OFFSET] = TEMP_R3; // SET HEATER AND BOOST LEVEL B[COFFEEPOT_DEVICE_P0 + HEATERREGISTER_OFFSET] = PAR2_R1; B[COFFEEPOT_DEVICE_P0 + HEATERBOOST_OFFSET] = PAR3_R2; // TURN ON HEAT (AND LED) TEMP_R3 = W[COFFEEPOT_DEVICE_P0 + CONTROLREGISTER_OFFSET]; TEMP_R4 = HEATERPOWER; TEMP_R3 = TEMP_R3 ¦ TEMP_R4; R4.L = lo(LED_HEAT_ON); R4.H = hi(LED_HEAT_ON); TEMP_R3 = TEMP_R3 ¦ TEMP_R4; W[COFFEEPOT_DEVICE_P0 + CONTROLREGISTER_OFFSET] = TEMP_R3; // UPDATE DISPLAY .extern __Z23UpdateSimulationDisplayv; CALL __Z23UpdateSimulationDisplayv; CALL __Z23UpdateSimulationDisplayv; UNLINK; _TurnOnHeat_ASM.END: RTS;

TURNONHEAT_CPP.CPP

void TurnOnHeat_CPP(COFFEEPOT_DEVICE *coffeePot_BaseAddress, unsigned char heaterLevel, unsigned char heaterBoost) { #define DO_CORRECT_WAY true #if DO_CORRECT_WAY #warning("Compiling Correct Code") unsigned short int currentRegisterValue = coffeePot_BaseAddress->controlRegister; unsigned short int updatedRegisterValue = currentRegisterValue ¦ HEATERPOWERON_BIT; updatedRegisterValue = updatedRegisterValue ¦ LED_HEATERPOWERON_BIT; // Set Water Flow Rate to 0 coffeePot_BaseAddress->waterInFlowRegister = 0; // Set Heat Level and Boost Level for Heater coffeePot_BaseAddress->heaterRegister = heaterLevel; coffeePot_BaseAddress->heaterBoostRegister = heaterBoost; UpdateSimulationDisplay(); // Turn on Heat coffeePot_BaseAddress->controlRegister = updatedRegisterValue; UpdateSimulationDisplay(); #else #warning("Compiling Incorrect Code") unsigned short int newValue = HEATERPOWERON_BIT ¦ LED_HEATERPOWERON_BIT; coffeePot_BaseAddress->controlRegister = newValue; UpdateSimulationDisplay(); coffeePot_BaseAddress->heaterRegister = heaterLevel; UpdateSimulationDisplay(); #endif }

TURNONCOFFEE_CPP.CPP

void TurnOnCoffee_CPP(COFFEEPOT_DEVICE * coffeePot_BaseAddress) { // BIT MASK FOR COFFEE POD unsigned short int currentRegisterValue = coffeePot_BaseAddress->controlRegister; unsigned short int updatedRegisterValue = currentRegisterValue ¦ COFFEE_INSERT_BIT; // TURN HEAT DOWN LOW coffeePot_BaseAddress->heaterBoostRegister = 1; coffeePot_BaseAddress->heaterRegister = 1; // TURN ON COFFEE coffeePot_BaseAddress->controlRegister = updatedRegisterValue; printf("\n\tCoffee Pod Initialized: Coffee Intensity 42/42\n\n"); UpdateSimulationDisplay(); }

PREPARETHECOFFEEPOT_CPP.CPP

void PrepareTheCoffeePot_CPP(COFFEEPOT_DEVICE * coffeePot_BaseAddress) { // BIT MASK FOR INITIALIZATION unsigned short int currentRegisterValue = coffeePot_BaseAddress->controlRegister; unsigned short int updatedRegisterValue = currentRegisterValue ¦ INITandSTAYPOWEREDON_BIT; updatedRegisterValue = updatedRegisterValue ¦ LED_POWERON_BIT; bool deviceReady = false; // TURN ON INIT BIT coffeePot_BaseAddress->controlRegister = updatedRegisterValue; UpdateSimulationDisplay(); // WAIT UNTIL DEVICE READY BIT IS SET while(!deviceReady) { UpdateSimulationDisplay(); if(((coffeePot_BaseAddress->controlRegister & DEVICE_READY_BIT_RO) ^ DEVICE_READY_BIT_RO) == 0x0) deviceReady = true; else deviceReady = false; } }